# Computer Games Development

# Project Report

# Year IV

# Danny Delaney

# C00286125

# 13/04/2026

Contents

Acknowledgements	2

Project Abstract	2

Project Introduction and/or Research Question	2

Research Question:	2

Literature Review	3

Relevant Work	3

Influence	3

Evaluation and Discussion	4

Project Milestones	4

Major Technical Achievements	4

Networked Video Delta Compression Algorithm	4

OS-Native Collision Engine	5

Entering Windows	5

Remote Protocol and Packet Structuring	5

Project Review	5

What went right?	5

What went wrong?	6

Alternative Approaches & Technology Implications	6

Future Work	6

Conclusions	6

References	6

# Acknowledgements

I would like to thank my Supervisor Omar Ali for his help and support, my lectures for the knowledge they provided, the open-source community, particularly the maintainers of the DXCam and pynput libraries, whose tools made this project possible.

# Project Abstract

This project is an augmented-reality platformer game that operates directly on the Windows operating system desktop. Unlike traditional video games confined to an isolated rendering window, this application transforms active, user-opened desktop Windows (such as Notepad or Chrome) into physical platforms that a virtual character can navigate, stand on, and interact with.

Operating on a multi-threaded Client-Server model, the system facilitates a multiplayer experience. A host machine (Sender) renders the local desktop physics environment and streams the visual output to a remote client (Receiver). The Receiver captures external inputs and transmits them back to the host, allowing remote manipulation of the desktop entity. To maintain a target framerate of 18 FPS without overwhelming system resources, a custom Networked Video Delta Compression algorithm was engineered. This algorithm utilizes Keyframe and Patch logic, isolating pixel-level changes between frames to transmit only localized updates rather than continuous 1080p full-frame captures.

The resulting system successfully blurs the boundary between the host operating system and the game engine, producing a dynamic, procedurally generated platforming environment dictated by the user's live workflow.

# Project Introduction and/or Research Question

The traditional paradigm of desktop computing heavily separates workspace from play space. Video games are executed within their own sandboxed memory and visual space, completely distinct from the operating system's graphical user interface. This project was made to break that split, drawing direct inspiration from interactive multimedia such as Alan Becker's "Animator vs. Animation" and the "Desktop Goose" desktop extension.

The core objective of this project is to make an interactive platforming environment onto a live Windows desktop environment, allowing an online counterpart to invade and interact with a host's workspace. The virtual Stickman character is rendered as a top-level, transparent overlay capable of traversing open application windows, executing specific animations (Idle, Run, Jump, Punch), and interacting with UI elements.

## Research Question:

Can a full game be made directly onto the desktop, integrated directly with the operating system allowing interaction with the windows while keeping everything usable, with the game player and the system user being on two separate machines connected over the internet.

To answer this question, the project established several primary objectives:

- Desktop Integration: Utilize the Windows API to get real-time window dimensions to act as platforms.

- Network Efficiency: Create a custom TCP structure and delta-frame compression to bypass the heavy CPU costs of continuous full-screen sending.

- Advanced Rendering: Create a system capable of rendering an entity seamlessly over the OS, while also allowing the entity to enter specific windows, capturing and blending the target application's device context.

- Procedural Workspaces: Implement a YAML-based layout manager capable of automatically spawning, positioning, and restoring applications to create playable levels out of generated windows.

# Literature Review

To enable a remote user (the Receiver) to play on a host's desktop (the Sender) over the internet, the system must overcome network delay while remaining lightweight enough to keep the host machine usable.

## Relevant Work

Research in cloud-based game streaming shows latency as the primary factor degrading "Quality of Experience" (QoE).

Projects like DeltaStream demonstrate that focusing on redundancy—sending only the changes between frames—can reduce bandwidth by up to 49% compared to full-frame captures for dynamic scenes.

## Influence

Inspired by these optimization strategies, this project implemented a custom Networked Video Delta Compression algorithm. To avoid the CPU heavy costs of full-screen streaming, the system uses Keyframe and Patch’s. By only getting pixel changes and transmitting patches, the project maintains a target of 18 FPS, ensuring the game remains playable even when network conditions are not ideal.

# Evaluation and Discussion

## Project Milestones

The project development lifecycle was divided into several key milestones:

- Desktop Capture over a TCP socket :The project started by using with PrintScreen for the full window it takes a screenshot of the screen to be converted to bytes and sent over the TCP socket where the Receiver would convert it to an image, then used the win32 API to print windows when entered to avoid having to send the full screen and allowing for windows to be captured even when behind other windows. I added patches to avoid sending parts of the screen that aren’t updated. Finally, I swapped to DXCam to take the prerender image off the GPU to improve speed.

- Physics and Stickman: I added a square instead of a stickman at first to keep the initial development simple and made simple physics such as jump and move. I converted it into a stickman and added punch interaction.

- Windows Interaction: I first used the Win32 API to find windows for collision interactions. I then allowed them to be entered and added click interactions. I added the ability to place an A in a notepad. Finally, I added the manager to setup windows layouts to place windows in the positions needed to make levels.

- Secondary Apps: I added two secondary apps one is the Launcher which launches the receiver and sender with the IP addresses The Second is the Manager which loads layouts placing windows in the correct places and at the correct sizes it also sends the locations of dots over the socket to be rendered on the display.

## Major Technical Achievements

### Networked Video Delta Compression Algorithm

To avoid the immense bandwidth required to stream uncompressed 1080p images, a patch-based differential algorithm was engineered. The Sender node evaluates the absolute difference between the current frame array and the previous frame array. The algorithm executes as follows:

- The raw absolute difference matrix is converted to a grayscale colour space.

- A strict threshold (DIFF_THRESH = 18) is applied to filter out minor rendering artifacts and other errors generating a binary mask.

- A buffer (DILATE_K = 7) thickens the mask to group proximate pixel changes into unified clusters.

- OpenCV contour mapping identifies the localized regions of change, converting them to discrete bounding rectangles.

- The _merge_rects function combines overlapping rectangles (with an 8-pixel gap tolerance) to prevent sending many separate packets.

- If the total area of the merged rectangles exceeds 25% of the total screen area (MAX_CHANGED_FRACTION = 0.25), the algorithm aborts patching and transmits a full Keyframe to prevent extra work. Otherwise, the subsets are sliced, JPEG encoded and transmitted as a custom multi-patch packet structure.

### OS-Native Collision Engine

The physical environment of the game is entirely made from live Windows OS data. The window_platforms.py module executes user32.EnumWindows to iterate through all GUI-based handles currently active in the system. The engine queries the RECT coordinates of these handles, converting them into dynamic collision boundaries.

Collision response relies on predictive intersection. For vertical collision, if the character is descending, the engine verifies if the previous foot coordinate was positioned above the platform's top edge, and the current foot coordinate is below it. If verified, the downward velocity is zeroed, the character's Y coordinate is snapped to the platform surface, and the on_ground state bool is triggered. Horizontal collisions apply similar logical constraints to the left and right window borders, preventing phasing and allowing wall-sliding states.

### Entering Windows

The Stickman can leave of the desktop and enter the localized space of a specific window. When a player presses down while resting on an active window top, the engine logs the specific HWND (Window Handle) target.

The main sender loop adapts. Instead of utilizing the DXCam global screen capture, it changes to the Win32 PrintWindow API to isolate and capture only the target windows context. A localized alpha-blending matrix operation is executed using NumPy. The sprite's 4-channel ARGB array is mathematically blended onto the 3-channel BGR application frame array based on the sprite's localized position offset against the window. This merged frame is then streamed by the standard delta-compression pipeline to the Receiver.

### Remote Protocol and Packet Structuring

To minimize latency overhead, Python's struct module is utilized to pack transmission data directly into Big-Endian C-style binary arrays.

- Control Packets (Client to Host): Maintain a static 12-byte footprint. Encoded as !iii representing horizontal velocity (dx), vertical velocity (dy), and a bool action trigger (click/punch).

- Video Patch Packets (Host to Client): Variable length packets beginning with an 8-bit P header, followed by a 16-bit unsigned short containing the patch count. Each patch has a 12-byte sub-header (X, Y, Width, Height, Payload Size) followed by the raw JPEG binary block. This strict packet guarantees that network fragmenting does not corrupt the OpenCV image.

# Project Review

## What went right?

The integration of ctypes for interacting with the Windows API was highly successful. Using GetWindowRect provided an extremely fast method for getting live platform terrain without noticeable drops in frame rate. The layouts.yaml and manager.py dynamic layout management architecture executed flawlessly, allowing the programmatic instantiation and precise grid-placement of terrain, seamlessly converting file execution into game level generation. Additionally, the rendering states (Idle, Run, Jump, Punch) scaled cleanly at the 320x320 pixel resolution.

## What went wrong?

I tried many ways to get the window sent at first starting with requesting a screen share using electron however it was inconsistent at best. I then tried using apache guacamole on a virtual machine but similarly it has hard to integrate into a project. And when I finally got a working display through python sending packages of the screen the frame rate was less than desired.

For a large amount of the project the stickman would always be rendered on top of applications causing an issue when an entered application was put at the back of the screen making the stickman look like he was walking on air on the local machine.

Finally, I wanted more interaction with specific windows, for example a more in-depth notepad interaction and possible CSV sheet interaction. The addition of more gameplay elements side scrolling levels, power ups and enemies was planned but time was too short.

## Alternative Approaches & Technology Implications

If continuing this system in the future, the network protocol architecture could be analysed. Currently, the video stream and the input controls are divided across two separate TCP sockets. Moving the Input Control to a UDP socket could reduce input latency. Given that input is transmitted at 60Hz, the dropped packets inherent to UDP would be negligible, whereas TCP's packet delivery guarantee introduces unnecessary wait for outdated data.

# Future Work

I plan on getting a set of virtual machines set up on a home computer acting as a cloud allowing anyone to join and mess with a desktop. Each VM will be factory reset allowing people to try break the computers as much as possible with the stickman.

I also plan on more level like gameplay dynamic windows moving with the stickman’s movement making side scrolling levels, powerups to allow extra interaction and simple enemies as a goal.

# Conclusions

The goal of this project was to determine if a fully functional, multiplayer game could be built directly into a live Windows desktop environment while maintaining the workspace's use for a host user. The successful development of this project shows that the traditional boundary between an operating system's GUI and a game engine's rendering space can be effectively blurred.

This project proves that augmented-reality desktop gaming is possible through high-performance screen capture and efficient networked inputs. Future iterations could further optimize the streaming latency and add more gameplay elements, but the current system provides a robust start for interactive, cross-machine desktop gaming (in the most literal sense).

# References

Software & Libraries

- Lin, Y. (2023). DXCam: A fast Python screen capture library for Windows using Desktop Duplication API. [Online]. Available at: https://github.com/ra1nty/DXCam.

- Palmér, M. (2022). pynput: Control and monitor input devices. [Online]. Available at: https://github.com/moses-palmer/pynput.

- Python Software Foundation. (2024). Python Language Reference & Standard Libraries (socket, ctypes). [Online]. Available at: https://www.python.org.

- Riverbank Computing. (2023). PyQt5: Python bindings for the Qt cross-platform application framework. [Online]. Available at: https://www.riverbankcomputing.com/software/pyqt/.

- Simonov, K. (2021). PyYAML: Canonical YAML parser and emitter for Python. [Online]. Available at: https://pyyaml.org/.

Technical Documentation

- Microsoft Corp. (2024). Windows Desktop Duplication API Documentation. [Online]. Available at: https://learn.microsoft.com/en-us/windows/win32/desktop-programming.

- YAML.org. (2021). YAML Ain’t Markup Language (YAML™) Version 1.2. [Online]. Available at: https://yaml.org/spec/1.2.2/.

Assets

- Gentry, J. (2014, March 18). Brick background. [Online]. Available at: https://www.publicdomainfiles.com/show_file.php?id=13951258814891.

- Hodan, G. (2014, April 4). Grass. [Online]. Available at: https://www.publicdomainfiles.com/show_file.php?id=13966569011044.

- Kratochvil, V. (2014, April 3). Tall office building. [Online]. Available at: https://www.publicdomainfiles.com/show_file.php?id=13965680411422.

- Octopyte. (2023). Stickman Animation Pack. [Online]. Available at: https://octopyte.itch.io/stickman-pack.

- Rgtaylor_csc. (2012, November 12). Illustrated cloud. [Online]. Available at: https://www.publicdomainfiles.com/show_file.php?id=13527842413455.